## Circuit Analysis (Lab1) Experiment1:IntroductiontoLabInstrumentsandComponents

Name: Maira		Registration No:2025-CE-79

Date: 4 –march -2026	Grade and Signature:

| CLO2 | Use with identification the circuit components, breadboards, multimeters, power supplies ,signal generators ,and oscilloscopes. |  |  |  |
| --- | --- | --- | --- | --- |
| Psychomotor/Affective | Level1 (1) | Level2 (2-3) | Level3 (4-5) | Level4 (6-7) |
| Report Marks(3) |  |  | Total marks (10) |  |

_

Objectives: This experiment will provide exposure to the various test equipment to be used in subsequent experiments. A primary purpose of this lab course is for you to master the use of electronic test equipment.

Post Lab Questions

- Define Oscilloscope and write its applications.

Oscilloscope:
An oscilloscope is an electronic measuring instrument used to display and analyze electrical signals         (voltages) in the form of a waveform on a screen with respect to time.

It shows how voltage changes over time.

### Applications

- Measuring voltage amplitude

- Observing signal waveform shape

- Measuring frequency and time period

- Checking noise or distortion in signals

- Testing electronic circuits

- Fault diagnosis in electronic equipment

- Measuring phase difference between signals

- What is Peak to Peak Voltages?

Peak-to-Peak Voltage (Vp) is the total voltage difference between the maximum positive peak and the maximum negative peak of a waveform.

Vp=Vmax−Vmin

For a symmetrical wave:

Vp=2Vpeak

It represents the full vertical height of the signal.

- What is RMS value?

RMS (Root Mean Square) value is the effective value of an alternating voltage or current that produces      the same heating effect as DC.

- Write the RMS values of Sine ,Square and Triangular wave?

| Waveform | RMS Value |
| --- | --- |
| Sine wave | ( V_{rms} = V_p \ sqrt{2}= 0.707Vp |
| Square wave | ( V_{rms} = V_p ) |
| Triangular wave | ( V_{rms} = V_p \sqrt{3} = 0.577Vp |

Explain DC offset ?

DC Offset is the amount by which an AC signal is shifted above or below the zero-voltage reference line.

- If waveform center is above zero → positive DC offset

- If below zero → negative DC offset